AIRMIND

Smart Environment AI-IoT Monitoring System

COMPLETE DEVELOPER HANDOFF DOCUMENT

| Project AirMind v2.0 | Version Release Candidate 1 |
| --- | --- |
| Institution RMKEC — March 2025 | Platform ESP32 + Python + Web + Mobile |

This document contains complete technical details for handoff to a new developer,
judge review, or system extension. All code, APIs, and configurations are included.

# 1. Project Overview

AirMind is a complete Smart Environment Monitoring and Air Control System built on an ESP32 microcontroller. It reads environmental and electrical data in real time, controls an air-output relay based on air quality and occupancy state, logs data continuously for AI/ML training, and exposes live sensor data and AI predictions through a Flask API consumed by both a PC web dashboard and a mobile app.

## 1.1 System Purpose

- Real-time monitoring of AQI, CO2, temperature, humidity, gas compounds, and electrical parameters

- Intelligent automation — AirCtrl output activated based on AQI thresholds and occupancy

- AI/ML pipeline predicting AQI and CO2 trends 10s, 30s, and 60s ahead

- Anomaly detection and intelligent action recommendations (KEEP_OFF / TURN_ON / RUN_2MIN / RUN_5MIN)

- Live web dashboard and mobile app displaying all sensor data and AI predictions

## 1.2 Demo Explanation (For Judges)

"This system uses an ESP32 to collect real-time environmental and energy data from multiple sensors including gas, temperature, humidity, occupancy, and electrical monitoring. The device performs local air-control automation and continuously exports structured data. A Python AI engine processes this data to forecast short-term AQI and CO2 trends, recommend intelligent air-control actions, and detect abnormal conditions. The dashboard and mobile app present both live measurements and AI predictions together in one interface."

# 2. System Architecture

The system is divided into 3 layers that communicate over Wi-Fi on the same local network:

| Layer 1 — ESP32 Hardware (Firmware) |  |
| --- | --- |
| Role | Reads all sensors every 2 seconds, controls AirCtrl relay, drives LCD, LED, buzzer |
| Language | C++ / Arduino Framework |
| File | smart_env_esp32.ino |
| Output | JSON POST to Flask API every 2 seconds over Wi-Fi |
| CSV | Also logs CSV lines to Serial for offline AI training |

| Layer 2 — Python AI/ML Backend (Flask) |  |
| --- | --- |
| Role | Receives ESP32 data, appends to CSV, runs AI models, serves predictions |
| Language | Python 3 |
| File | ai_api.py |
| Port | 5000 |
| Routes | POST /data GET /live GET /predict |

| Layer 3 — Frontend (Web + Mobile) |  |
| --- | --- |
| Role | Displays live sensor data and AI predictions in real time |
| Web file | dashboard.html — open in any browser |
| Mobile file | mobile_app.html — open on phone browser |
| Data source | Polls /live and /predict every 3 seconds |
| Fallback | Shows simulated data if API unreachable (demo mode) |

# 3. Hardware & Pin Mapping

| Component | Status | Notes |
| --- | --- | --- |
| MQ135 Gas Sensor | GPIO 34 | AQI and CO2 approximation — ADC input |
| PIR Motion Sensor | GPIO 27 | Occupancy detection |
| AirCtrl / Relay | GPIO 12 | Controls air output — HIGH = ON |
| Buzzer | GPIO 14 | Alert tones based on AQI level |
| Red LED | GPIO 25 | AQI > 100 indicator |
| Green LED | GPIO 26 | AQI <= 50 indicator |
| Blue LED | GPIO 33 | AQI 51-100 indicator |
| DHT11 Sensor | GPIO 4 | Temperature and humidity |
| I2C SDA | GPIO 21 | LCD and INA219 data line |
| I2C SCL | GPIO 22 | LCD and INA219 clock line |
| INA219 Power Sensor | I2C 0x40 | Voltage, current, power, energy |
| LCD 16x2 I2C | I2C 0x27 | Display — 3 rotating pages of data |

## 3.1 Required Libraries (Arduino IDE)

- Adafruit_INA219 — install via Library Manager

- DHT sensor library by Adafruit

- LiquidCrystal_I2C by Frank de Brabander

- ArduinoJson by Benoit Blanchon (v6+)

- WiFi.h — built into ESP32 board package

- HTTPClient.h — built into ESP32 board package

- Preferences.h — built into ESP32 board package

# 4. ESP32 Firmware

## 4.1 Configuration — 3 Lines to Change

Before flashing, update only these 3 lines in smart_env_esp32.ino:

const char* ssid     = "YOUR_WIFI_NAME";      // Your WiFi network name

const char* password = "YOUR_WIFI_PASSWORD";  // Your WiFi password

const uint8_t bssid[6] = {0xC6,0x31,0x37,0xD9,0x09,0xC6}; // Your router BSSID

const char* serverUrl = "http://10.76.154.13:5000/data";   // Your laptop IP

To find your laptop IP: open CMD and run ipconfig — look for IPv4 Address under Wi-Fi.

To find BSSID: open CMD and run: netsh wlan show interfaces — look for BSSID line.

## 4.2 AirCtrl Logic (DO NOT CHANGE)

| Air Control Activation Rules |  |
| --- | --- |
| Rule 1 | occupancy = 1 AND AQI > 100 → AirCtrl ON |
| Rule 2 | occupancy = 0 AND AQI > 200 → AirCtrl ON (emergency) |
| Rule 3 | All other cases → AirCtrl OFF |

## 4.3 Occupancy Modes

- — PIR sensor controls occupancy. Motion detected = occupied for 120 seconds

- — Occupancy forced to 1 permanently. Used for testing/demo

- — type AUTO or MANUAL and press Enter

## 4.4 Serial Commands

| Available Serial Monitor Commands (115200 baud) |  |
| --- | --- |
| HELP | Lists all available commands |
| STATUS | Prints full sensor report to serial |
| AUTO | Switch to AUTO occupancy mode (PIR) |
| MANUAL | Switch to MANUAL mode (occupancy forced = 1) |
| SHOW_R0 | Display current saved R0 calibration value |
| SAVE_R0=7.20 | Save new R0 value for MQ135 calibration |
| CAL_ON | Enable calibration mode — shows R0 estimation |
| CAL_OFF | Disable calibration mode — return to normal |

# 5. Python Backend (ai_api.py)

## 5.1 Installation

Install all required packages with one command:

py -m pip install pandas numpy scikit-learn joblib flask flask-cors

## 5.2 Starting the Server

cd "C:\PROJECTS\RMKEC MARCH 10\LASTSTAGE\BESTAIML"

py ai_api.py

## 5.3 API Routes

| Flask API Endpoints — All on Port 5000 |  |
| --- | --- |
| POST /data | ESP32 sends sensor JSON here every 2 seconds |
| GET /live | Dashboard/app fetches current sensor values |
| GET /predict | Dashboard/app fetches AI predictions and recommendations |

## 5.4 /predict JSON Response Format

{"aqi_now": 205.0,   "aqi_10sec": 209.84,  "aqi_30sec": 210.34,  "aqi_60sec": 211.15,

"co2_now": 1580.0,  "co2_10sec": 1608.30, "co2_30sec": 1612.35, "co2_60sec": 1646.40,

"action": "RUN_5MIN",  "anomaly": "NORMAL"}

## 5.5 /live JSON Response Format

{"aqi": 205, "airQuality": "Very Unhealthy", "co2": 1580, "co2Status": "High CO2 Level",

"temp": 31.4, "humidity": 66.8, "nh3": 80.0, "benzene": 8.5, "co": 2.0,

"voltage": 11.92, "current": 412.75, "power": 4.92, "energy": 0.003512,

"cost": 0.0281, "occupancy": 1, "airCtrl": 1, "mode": "AUTO"}

## 5.6 AI Model Files Required

These .pkl files must be in the same folder as ai_api.py:

- model_aqi_10sec.pkl — AQI prediction +10 seconds

- model_aqi_30sec.pkl — AQI prediction +30 seconds

- model_aqi_60sec.pkl — AQI prediction +60 seconds

- model_co2_10sec.pkl — CO2 prediction +10 seconds

- model_co2_30sec.pkl — CO2 prediction +30 seconds

- model_co2_60sec.pkl — CO2 prediction +60 seconds

- model_action.pkl — Action recommendation model

- model_anomaly.pkl — Anomaly detection model (IsolationForest)

Train/retrain models by running:

py air_ai_ml.py

# 6. Web Dashboard & Mobile App

## 6.1 Files

| Frontend Files |  |
| --- | --- |
| dashboard.html | Full PC web dashboard — open by double clicking the file |
| mobile_app.html | Mobile app UI — open in phone browser using laptop IP |

## 6.2 What the Dashboard Shows

| Dashboard Sections |  |
| --- | --- |
| Live Readings | AQI, CO2, Temperature, Humidity, AirCtrl status, Mode, Occupancy |
| Trend Charts | AQI sparkline and CO2 sparkline — last 30 readings (2s interval) |
| AI Predictions | AQI and CO2 forecasts at NOW / +10s / +30s / +60s |
| AI Recommendation | KEEP_OFF / TURN_ON / RUN_2MIN / RUN_5MIN with description |
| Anomaly Detection | NORMAL or ANOMALY status from IsolationForest model |
| Sensor Array | NH3, Benzene, CO, Voltage, Current, Power with progress bars |
| Energy Section | Energy kWh, cost in INR, CO2 emitted in grams |
| Carbon Equivalent | Tree icons showing tree-minutes needed to offset emissions |
| Occupancy Panel | Person icon, AUTO/MANUAL mode toggle, motion status |
| AirCtrl Panel | Spinning fan animation when ON, relay status, activation reason |
| History Chart | 60-reading timeline switchable between AQI/CO2/Temp/Humidity |

## 6.3 Connecting to Real ESP32 Data

The dashboard automatically tries to fetch from these URLs every 3 seconds:

http://127.0.0.1:5000/live     (live sensor data)

http://127.0.0.1:5000/predict  (AI predictions)

If the API is unreachable, it automatically falls back to simulated demo data so the UI always looks live. When ai_api.py is running and ESP32 is connected, real data replaces the simulation automatically.

## 6.4 Opening on Mobile Phone

The mobile app must use your laptop IP (not 127.0.0.1) since the phone is a different device:

- Find laptop IP by running ipconfig in CMD

- In mobile_app.html, change 127.0.0.1 to your laptop IP (e.g. 10.76.154.13)

- Open mobile_app.html on phone browser OR serve it via Flask

To serve via Flask (easiest for phone), add this route to ai_api.py:

from flask import send_file

@app.route('/app')

def serve_app(): return send_file('mobile_app.html')

Then phone opens: http://10.76.154.13:5000/app

# 7. Complete Data Flow

| End-to-End Data Flow |  |
| --- | --- |
| Step 1 | ESP32 reads MQ135, DHT11, INA219, PIR every 2 seconds |
| Step 2 | ESP32 smooths readings using 10-sample moving average |
| Step 3 | ESP32 calculates AQI, CO2 approximation, gas levels, energy |
| Step 4 | ESP32 applies AirCtrl logic and controls relay output |
| Step 5 | ESP32 sends JSON to http://LAPTOP_IP:5000/data via HTTP POST |
| Step 6 | Flask /data route receives JSON, stores in memory, appends to CSV |
| Step 7 | Dashboard fetches /live every 3 seconds — displays sensor values |
| Step 8 | Dashboard fetches /predict every 3 seconds — runs AI models on CSV |
| Step 9 | AI models return forecasts, action recommendation, anomaly status |
| Step 10 | Dashboard updates all charts, cards, and indicators in real time |

## 7.1 CSV Data Format

The CSV file (air_quality_data.csv) is automatically created and updated by ai_api.py. Each row contains:

time_s, aqi, co2, temp, humidity, nh3, benzene, co,

busVoltage, current_mA, power_W, energy_kWh, cost,

occupancy, airCtrl, mode, dhtErr, inaErr, mqErr

# 8. Demo Startup Guide

## Step-by-Step Demo Launch

- Open CMD in project folder and run: py ai_api.py  — keep this window open at all times

- Power on ESP32  — it connects to Wi-Fi and starts sending data in ~10 seconds

- Watch CMD for: [ESP32] AQI=... CO2=... Temp=...  — confirms connection

- Double click dashboard.html  — live data appears within 3 seconds

- Open mobile_app.html on phone browser  — use laptop IP in URL

## 8.1 Verification Checklist

| Component | Status | Notes |
| --- | --- | --- |
| ai_api.py running | ✅ DONE | CMD shows Flask startup message |
| Port 5000 accessible | ✅ DONE | http://127.0.0.1:5000/live returns JSON |
| ESP32 connected | ✅ DONE | CMD shows [ESP32] lines every 2 seconds |
| Dashboard showing real data | ✅ DONE | AQI/CO2 values change in real time |
| AI predictions working | ✅ DONE | http://127.0.0.1:5000/predict returns forecasts |
| Mobile app working | ✅ DONE | Phone browser shows live data |

# 9. Troubleshooting

| Common Issues and Fixes |  |
| --- | --- |
| Dashboard shows random/demo data | ai_api.py is not running OR browser cannot reach port 5000. Run py ai_api.py and try http://127.0.0.1:5000/live in browser |
| Browser shows ERR_CONNECTION_REFUSED | Flask is not running. Open CMD, cd to project folder, run py ai_api.py |
| ESP32 not sending (no [ESP32] lines in CMD) | Check WiFi credentials in firmware, check laptop IP is correct in serverUrl, ensure same Wi-Fi network |
| /predict returns error about missing features | Run py air_ai_ml.py to retrain models. Ensure CSV has at least 50 rows first |
| Port 5000 blocked by firewall | Run in Admin CMD: netsh advfirewall firewall add rule name=Flask dir=in action=allow protocol=TCP localport=5000 |
| Laptop IP changes every session | Run ipconfig before each session to get current IP. Update serverUrl in ESP32 and re-flash |
| INA219 not found on startup | Check I2C wiring on GPIO 21 (SDA) and GPIO 22 (SCL). Verify I2C address 0x40 |
| DHT11 reading errors | Check wiring on GPIO 4. DHT11 needs a 10k pull-up resistor on data line |

# 10. Complete File Structure

| Project Folder Contents |  |
| --- | --- |
| smart_env_esp32.ino | ESP32 firmware — flash to hardware |
| ai_api.py | Flask API server — run with py ai_api.py |
| air_ai_ml.py | AI/ML training script — run to generate .pkl files |
| air_quality_data.csv | Auto-generated by API — training data from ESP32 |
| dashboard.html | Web dashboard — open in browser |
| mobile_app.html | Mobile app — open on phone browser |
| model_aqi_10sec.pkl | Trained AQI +10s prediction model |
| model_aqi_30sec.pkl | Trained AQI +30s prediction model |
| model_aqi_60sec.pkl | Trained AQI +60s prediction model |
| model_co2_10sec.pkl | Trained CO2 +10s prediction model |
| model_co2_30sec.pkl | Trained CO2 +30s prediction model |
| model_co2_60sec.pkl | Trained CO2 +60s prediction model |
| model_action.pkl | Trained action recommendation model |
| model_anomaly.pkl | Trained anomaly detection model |

# 11. Completion Status

| Component | Status | Notes |
| --- | --- | --- |
| ESP32 Hardware | ✅ DONE | All sensors wired and tested |
| ESP32 Firmware | ✅ DONE | Smoothing, control logic, Wi-Fi POST |
| CSV Data Logging | ✅ DONE | Auto-appended by Flask on every POST |
| AI Model Training | ✅ DONE | 8 .pkl files generated and tested |
| Flask API Server | ✅ DONE | 3 routes: /data /live /predict |
| Web Dashboard | ✅ DONE | Full UI with charts, AI panel, carbon tracking |
| Mobile App | ✅ DONE | Phone-optimized UI with same data |
| Live Data Integration | ✅ DONE | Real ESP32 data flowing to dashboard |
| AI Predictions on UI | ✅ DONE | Forecasts, action, anomaly displayed |
| Carbon/Tree Equivalent | ✅ DONE | CO2 emission calculator with tree icons |
| Energy Monitoring | ✅ DONE | kWh, cost in INR, CO2 emitted |
| Anomaly Detection | ✅ DONE | IsolationForest — NORMAL/ANOMALY status |

System is fully operational and ready for demo. All components tested and working.